CLOCKWISE ROTATION OF STEPPER MOTOR USING 8051 USING PROTEUS

AIM:

To write an assembly language program to rotate the Stepper Motor in clockwise direction in 8051 using Proteus

SOFTWARE REQUIRED:

- Proteus 8 software.

PROGRAM:

ORG 0000H         ; Starting address

UP:

MOV P2, #08H  ; Step 1: Coil 4 = ON, others OFF (Clockwise direction)

ACALL DELAY   ; Delay for a short period

MOV P2, #04H  ; Step 2: Coil 3 = ON, others OFF

ACALL DELAY   ; Delay for a short period

MOV P2, #02H  ; Step 3: Coil 2 = ON, others OFF

ACALL DELAY   ; Delay for a short period

MOV P2, #01H  ; Step 4: Coil 1 = ON, others OFF

ACALL DELAY   ; Delay for a short period

SJMP UP       ; Repeat the sequence for continuous rotation

DELAY:

MOV R4, #18   ; Outer delay loop counter

H1:

MOV R3, #255  ; Inner delay loop counter

H2:

DJNZ R3, H2   ; Decrement and loop if not zero

DJNZ R4, H1   ; Decrement and loop if not zero

RET           ; Return from delay subroutine

END

CIRCUIT DIAGRAM:

OUTPUT:

The stepper motor is rotating in clockwise direction in steps.

RESULT:

Thus, the program has been successfully verified and executed.