DIGITAL CLOCK ON LCD

Team 5 : A. Neha  192225026  , R. Varshitha  192225072

AIM:

To write an assembly language program to display digital clock on LCD with using Proteus.

SOFTWARES REQUIRED:

- Proteus software

PROGRAM:

ORG 0000H               ; Start address of the program

MOV R7, #00H            ; Initialize hours (HH)

MOV R6, #00H            ; Initialize minutes (MM)

MOV R5, #00H            ; Initialize seconds (SS)

ACALL INIT_LCD          ; Initialize the LCD

MAIN_LOOP:

ACALL UPDATE_LCD       ; Update the time on the LCD

ACALL DELAY_1_SEC      ; Wait for 1 second

ACALL INCREMENT_TIME   ; Increment time (HH:MM:SS)

SJMP MAIN_LOOP         ; Repeat the process

; Subroutine to initialize the LCD

INIT_LCD:

MOV R1, #38H

ACALL CMD_WRITE

ACALL SHORT_DELAY       ; Short delay between commands

MOV R1, #0CH            ; Display ON, Cursor OFF

ACALL CMD_WRITE

ACALL SHORT_DELAY

MOV R1, #06H            ; Auto-increment cursor

ACALL CMD_WRITE

ACALL SHORT_DELAY

MOV R1, #01H            ; Clear display

ACALL CMD_WRITE

ACALL DELAY_CLEAR       ; Longer delay after clearing display

RET

; Subroutine to increment time

INCREMENT_TIME:

INC R5                 ; Increment seconds (SS)

CJNE R5, #60, DONE_SEC ; If seconds < 60, continue

MOV R5, #00H           ; Reset seconds to 00

INC R6                 ; Increment minutes (MM)

CJNE R6, #60, DONE_SEC ; If minutes < 60, continue

MOV R6, #00H           ; Reset minutes to 00

INC R7                 ; Increment hours (HH)

CJNE R7, #24, DONE_SEC ; If hours < 24, continue

MOV R7, #00H           ; Reset hours to 00

DONE_SEC:

RET

; Subroutine to update the LCD with the current time

UPDATE_LCD:

MOV R1, #80H            ; Move cursor to the first line of the LCD

ACALL CMD_WRITE

ACALL SHORT_DELAY

MOV A, R7               ; Load hours (HH) into accumulator

ACALL DISPLAY_TWO_DIGIT ; Display hours (HH)

ACALL DISPLAY_COLON     ; Display ':'

MOV A, R6               ; Load minutes (MM) into accumulator

ACALL DISPLAY_TWO_DIGIT ; Display minutes (MM)

ACALL DISPLAY_COLON     ; Display ':'

MOV A, R5               ; Load seconds (SS) into accumulator

ACALL DISPLAY_TWO_DIGIT ; Display seconds (SS)

RET

; Subroutine to display two-digit numbers on the LCD

DISPLAY_TWO_DIGIT:

MOV B, #10              ; Divide the value in A by 10

DIV AB                  ; Quotient in A (tens), remainder in B (ones)

ADD A, #30H             ; Convert tens digit to ASCII

ACALL DISPLAY_CHAR      ; Display the tens digit

MOV A, B                ; Move the remainder (ones digit) to A

ADD A, #30H             ; Convert ones digit to ASCII

ACALL DISPLAY_CHAR      ; Display the ones digit

RET

; Subroutine to display colon ':' on the LCD

DISPLAY_COLON:

MOV A, #3AH             ; ASCII value of ':'

ACALL DISPLAY_CHAR      ; Display ':'

RET

; Subroutine to display a character on the LCD

DISPLAY_CHAR:

MOV P2, A               ; Send ASCII character to data pins (P2 connected to D0-D7 of LCD)

SETB P3.2               ; Set RS to 1 (data register)

CLR P3.3                ; Set RW to 0 (write mode)

SETB P3.4               ; Set E to 1 (Enable high)

NOP                     ; Small delay

CLR P3.4                ; Set E to 0 (Enable low)

ACALL SHORT_DELAY       ; Short delay after writing character

RET

; Subroutine to write command to the LCD

CMD_WRITE:

MOV P2, R1              ; Send command from R1 to data pins (P2 connected to D0-D7 of LCD)

CLR P3.2                ; Set RS to 0 (command register)

CLR P3.3                ; Set RW to 0 (write mode)

SETB P3.4               ; Set E to 1 (Enable high)

NOP                     ; Small delay

CLR P3.4                ; Set E to 0 (Enable low)

ACALL SHORT_DELAY       ; Short delay after writing command

RET

; Short delay subroutine (for small delays between LCD commands)

SHORT_DELAY:

MOV R2, #50             ; Adjust for a small delay

DELAY1:

DJNZ R2, DELAY1

RET

; Longer delay subroutine for clearing the LCD

DELAY_CLEAR:

MOV R2, #200            ; Longer delay after clearing the display

DELAY2:

DJNZ R2, DELAY2

RET

; Subroutine for 1-second delay

DELAY_1_SEC:

MOV R3, #50             ; Adjust this value to create a 1-second delay

DELAY_LOOP:

DJNZ R3, DELAY_LOOP     ; Simple delay loop

RET

END

CIRCUIT DIAGRAM:

OUTPUT:

- When this program is run, the LCD will display the current time in the format HH:MM.

- Every second, the display will update to increment the seconds value.

- After reaching 59 seconds, the seconds will reset to 00, and the minutes will increment.

- Similarly, when the minutes reach 59 and increment again, they will reset to 00, and the hours will increment.

- The hours will increment from 00 to 23 in a 24-hour format. When the hours reach 23 and the next second occurs, the hours, minutes, and seconds will all reset to 00:00:00.

RESULT:

Thus, the assembly language program to display digital clock on LCD with using Proteus was executed.