FADING OF AN LED USING ARDUINO

Date:

Aim:

To interface an LED with Arduino and to write a program to make the LED fade.

Components required:

- PC or laptop

- Arduino UNO

- 220  resistor

- LED

- Breadboard

- Connecting wires

Software required:

- Arduino IDE

Connections:

- Connect the longer leg of the LED, that is the anode, to the digital pin 9 in the Arduino, through a 220  resistor.

- Connect the shorter leg of the LED, that is the cathode, to the ground (GND) pin on the Arduino.

Procedure:

- Connect the components on a breadboard as mentioned above and connect the Arduino to the PC/laptop.

- Open Arduino IDE and open a new editor.

- Enter the required code save it as a “ino” file.

- In the “Tools” menu go to the “Port” tab and select the appropriate port.

- Upload the program into the Arduino and verify the output.

Program:

int led = 9;           // the PWM pin the LED is attached to
int brightness = 0;    // how bright the LED is
int fadeAmount = 5;    // how many points to fade the LED by
// the setup routine runs once when you press reset:
void setup() {
  // declare pin 9 to be an output:
  pinMode(led, OUTPUT);
}
// the loop routine runs over and over again forever:
void loop() {
  // set the brightness of pin 9:
  analogWrite(led, brightness);
  // change the brightness for next time through the loop:
  brightness = brightness + fadeAmount;
  // reverse the direction of the fading at the ends of the fade:
  if (brightness <= 0 || brightness >= 255) {
    fadeAmount = -fadeAmount;
  }
  // wait for 30 milliseconds to see the dimming effect
  delay(30);
}

Result:

A program has been written to make the LED connected to an Arduino fade and the output verified.