GENERATION OF SQUARE WAVE USING PROTEUS

AIM:

To write an assembly language program to generate square wave using 8051.

SOFTWARE REQUIRED:

- Proteus 8 software.

PROGRAM

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1: MOV R3,#255

H2: DJNZ R3,H2

DJNZ R4,H1

RET

END

CIRCUIT DIAGRAM:

- Adjust the frequency in horizontal and channel A to Observe the square waveform on the oscilloscope for a consistent, symmetrical shape.